=============================================================================

List of generated methods in project: DC17

THIS TEXT DESCRIPTION IS GENERATED BY THE TOOL. DO NOT MODIFY IT.

=============================================================================

Module "PWMRed"    (bean PWMJoe)

- PWMRed_Enable -This method enables the bean - it starts the signal generation. Events may be generated

(<DisableEvent>/<EnableEvent>).

- PWMRed_Disable -This method disables the bean - it stops the signal generation and events calling. When the timer is

disabled, it is possible to call <ClrValue> and <SetValue> methods.

- PWMRed_SetRatio16 -This method sets a new duty-cycle ratio. Ratio is expressed as a 16-bit unsigned integer number. 0 -

FFFF value is proportional to ratio 0 - 100%. The method is available only if it is not selected list of

predefined values in <Starting pulse width> property.   Note: Calculated duty depends on the timer

possibilities and on the selected period.

Module "PWMGreen"  (bean PWMJoe)

- PWMGreen_Enable -This method enables the bean - it starts the signal generation. Events may be generated

(<DisableEvent>/<EnableEvent>).

- PWMGreen_Disable -This method disables the bean - it stops the signal generation and events calling. When the timer is

disabled, it is possible to call <ClrValue> and <SetValue> methods.

- PWMGreen_SetRatio16 -This method sets a new duty-cycle ratio. Ratio is expressed as a 16-bit unsigned integer number. 0 -

FFFF value is proportional to ratio 0 - 100%. The method is available only if it is not selected list of

predefined values in <Starting pulse width> property.   Note: Calculated duty depends on the timer

possibilities and on the selected period.

Module "PWMBlue"   (bean PWMJoe)

- PWMBlue_Enable -This method enables the bean - it starts the signal generation. Events may be generated

(<DisableEvent>/<EnableEvent>).

- PWMBlue_Disable -This method disables the bean - it stops the signal generation and events calling. When the timer is

disabled, it is possible to call <ClrValue> and <SetValue> methods.

- PWMBlue_SetRatio16 -This method sets a new duty-cycle ratio. Ratio is expressed as a 16-bit unsigned integer number. 0 -

FFFF value is proportional to ratio 0 - 100%. The method is available only if it is not selected list of

predefined values in <Starting pulse width> property.   Note: Calculated duty depends on the timer

possibilities and on the selected period.

Module "MICOUT"    (bean ADC)

- MICOUT_Enable -Enables A/D converter bean. <Events> may be generated (<DisableEvent>/<EnableEvent>). If possible, this

method switches on A/D converter device, voltage reference, etc.

- MICOUT_Disable -Disables A/D converter bean. No <events> will be generated. If possible, this method switches off A/D

converter device, voltage reference, etc.

- MICOUT_Measure -This method performs one measurement on all channels that are set in the bean inspector. (Note: If the

<number of conversions> is more than one the conversion of A/D channels is performed specified number of times.)

- MICOUT_GetValue16 -This method returns the last measured values of all channels justified to the left. Compared with

<GetValue> method this method returns more accurate result if the <number of conversions> is greater than 1 and

<AD resolution> is less than 16 bits. In addition, the user code dependency on <AD resolution> is eliminated.

Module "MICPWR"    (bean BitIO)

- MICPWR_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- MICPWR_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if

the direction = _[output]_ or _[input/output]_.

Module "TI0"       (bean TimerInt)

- TI0_Enable -This method enables the bean - it starts the timer. Events may be generated (<DisableEvent>/<EnableEvent>).

- TI0_Disable -This method disables the bean - it stops the timer. No events will be generated.

Module "TI1"       (bean TimerInt)

- TI1_Enable -This method enables the bean - it starts the timer. Events may be generated (<DisableEvent>/<EnableEvent>).

- TI1_Disable -This method disables the bean - it stops the timer. No events will be generated.

Module "TIPIT"     (bean TimerInt)

- TIPIT_Enable -This method enables the bean - it starts the timer. Events may be generated (<DisableEvent>/<EnableEvent>).

- TIPIT_Disable -This method disables the bean - it stops the timer. No events will be generated.

Module "DFR1"      (bean DSP_Func_DFR)

- DFR1_dfr16FIR -The method computes a Finite Impulse Response, (FIR), filter for a vector of fractional data values.

Prior to any call to fir, the FIR filter data structure referred to by the first parameter pFIR must be

allocated and initialized. This allocation and initialization can be done either statically by declaring a

dfr16_tFirStruct data structure and then initializing it with a call to dfr16FIRInit, or dynamically by calling

dfr16FIRCreate.

- DFR1_dfr16FIRHistory -The method reinitializes the Finite Impulse Response, (FIR), filter history buffer data values.

Prior to any call to firHistory, the FIR filter must be initialized via a call to firCreate. The firHistory

function uses the private data structure established by firCreate to establish the past history of data

elements required by the FIR filter computation. The argument pX must point to n fractional data elements,

where n is the number of coefficients used to establish the FIR filter in the call to firCreate.

- DFR1_dfr16FIRInit -The method initializes the fir filter function from a previously allocated dfr16_tFirStruct data

structure. Typically, this dfr16_tFirStruct data structure is allocated statically by the caller prior to the

firInit call. However, firCreate also uses firInit to initialize the dfr16_tFirStruct that it dynamically

allocates.

- DFR1_dfr16FIRs -The method computes a Finite Impulse Response (FIR) filter for a single fractional data value.   Prior to

any call to fir, the FIR filter data structure referred to by the first parameter pFIR must be allocated and

initialized. This allocation and initialization can be done either statically by declaring a dfr16_tFirStruct

data structure and then initializing it with a call to dfr16FIRInit, or dynamically by calling dfr16FIRCreate.

- DFR1_dfr16FIRInt -The method computes an interpolating Finite Impulse Response, (FIR), filter for a vector of fractional

data values.   Prior to any call to firint, the FIR filter data structure referred to by the first parameter

pFIRINT must be allocated and initialized. This allocation and initialization can be done either statically by

declaring a dfr16_tFirIntStruct data structure and then initializing it with a call to dfr16FIRIntInit, or

dynamically by calling dfr16FIRIntCreate.

- DFR1_dfr16FIRIntInit -The method initializes the firint filter function from a previously allocated dfr16_tFirIntStruct

data structure. Typically, this dfr16_tFirIntStruct data structure is allocated statically by the caller prior

to the firintInit call. However, firintCreate also uses firintInit to initialize the dfr16_tFirIntStruct that

it dynamically allocates.

- DFR1_dfr16FIRDec -The method computes a decimating Finite Impulse Response, (FIR), filter for a vector of fractional data

values.   Prior to any call to firdec, the FIR filter data structure referred to by the first parameter pFIRDEC

must be allocated and initialized. This allocation and initialization can be done either statically by

declaring a dfr16_tFirDecStruct data structure and then initializing it with a call to dfr16FIRDecInit, or

dynamically by calling dfr16FIRDecCreate.

- DFR1_dfr16FIRDecInit -The method initializes the firdec filter function from a previously allocated dfr16_tFirDecStruct

data structure. Typically, this dfr16_tFirDecStruct data structure is allocated statically by the caller prior

to the firdecInit call. However, firdecCreate also uses firdecInit to initialize the dfr16_tFirDecStruct that

it dynamically allocates.

- DFR1_dfr16IIR -The method computes an Infinite Impulse Response, (IIR), filter for a vector of fractional data values

using a cascade filter of biquad coefficients.   Prior to any call to iir, the IIR filter data structure

referred to by the first parameter pIIR must be allocated and initialized. This allocation and initialization

can be done either statically by declaring a dfr16_tIirStruct data structure and then initializing it with a

call to dfr16IIRInit, or dynamically by calling dfr16IIRCreate.

- DFR1_dfr16IIRInit -The method performs the initialization for the cascade iir filter function and initializes the function

from a previously-allocated dfr16_tIirStruct data structure. Typically, the dfr16_tIirStruct data structure is

allocated statically by the caller prior to the iirInit call. The iirCreate function also uses iirInit to

initialize the dfr16_tIirStruct that it dynamically allocates.

- DFR1_dfr16AutoCorr -The method computes the first nz points of auto-correlation of a vector of fractional data values.

- DFR1_dfr16Corr -The method computes the full length correlation of two vectors of fractional data values.

- DFR1_dfr16CFFT -The method computes an in place, radix-2, complex, decimation-in-time, (DIT), forward Fast Fourier

Transform function for a vector of complex fractional data values.   Prior to any call to cfft, the CFFT data

structure referred to by the first parameter pCFFT must be allocated and initialized. This allocation and

initialization can be done either statically by declaring a dfr16_tCFFTStruct data structure and then

initializing it with a call to dfr16CFFTInit, or dynamically by calling dfr16CFFTCreate.

- DFR1_dfr16CFFTInit -The method performs the initialization for an in place, radix-2, complex, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, cfft, using complex fractional data values. Initializes the

function from a previously-allocated dfr16_tCFFTStruct data structure. Typically, the dfr16_tCFFTStruct data

structure is allocated statically by the caller prior to the cfftInit call. The cfftCreate function also uses

cfftInit to initialize the dfr16_tCFFTStruct that it dynamically allocates.

- DFR1_dfr16CIFFT -The method computes an in place, radix-2, complex, decimation-in-time (DIT), inverse Fast Fourier

Transform function for a vector of complex fractional data values.   Prior to any call to cifft, the CFFT data

structure referred to by the first parameter pCIFFT must be allocated and initialized. This allocation and

initialization can be done either statically by declaring a dfr16_tCFFTStruct data structure and then

initializing it with a call to dfr16CIFFTInit, or dynamically by calling dfr16CIFFTCreate.

- DFR1_dfr16CIFFTInit -The method performs the initialization for an in place, radix-2, complex, decimation-in-time (DIT),

inverse Fast Fourier Transform, (IFFT), using complex fractional data values. Initializes the function from a

previously-allocated dfr16_tCFFTStruct data structure. Typically, the dfr16_tCFFTStruct data structure is

allocated statically by the caller prior to the cifftInit call. The cifftCreate function also uses cifftInit to

initialize allocate the dfr16_tCFFTStruct dynamically.

- DFR1_dfr16RFFT -The method computes an in place, radix-2, real, decimation-in-time (DIT) forward Fast Fourier Transform

function for a vector of fractional data values.   Prior to any call to rfft, the RFFT data structure referred

to by the first parameter pRFFT must be allocated and initialized. This allocation and initialization can be

done either statically by declaring a dfr16_tRFFTStruct data structure and then initializing it with a call to

dfr16RFFTInit, or dynamically by calling dfr16RFFTCreate.

- DFR1_dfr16RFFTInit -The method performs the initialization for an in place, radix-2, real, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, rfft, using complex fractional data values. Initializes the

function from a previously allocated dfr16_tRFFTStruct data structure. Typically, the dfr16_tRFFTStruct data

structure is allocated statically by the caller prior to the rfftInit call. The rfftCreate function also uses

rfftInit to initialize the dfr16_tRFFTStruct that it dynamically allocates.

- DFR1_dfr16RIFFT -The method computes an in place, radix-2, decimation-in-time, (DIT), inverse Fast Fourier Transform

function for a structure of complex data values.   Prior to any call to rifft, the RFFT data structure referred

to by the first parameter pRIFFT must be allocated and initialized. This allocation and initialization can be

done either statically by declaring a dfr16_tRFFTStruct data structure and then initializing it with a call to

dfr16RIFFTInit, or dynamically by calling dfr16RIFFTCreate.

- DFR1_dfr16RIFFTInit -The method performs the initialization for an in place, radix-2, real, decimation-in-time (DIT),

forward Fast Fourier Transform (FFT) function, rifft, using complex fractional data values. Initializes the

function from a previously allocated dfr16_tRIFFTStruct data structure. Typically, the dfr16_tRIFFTStruct data

structure is allocated statically by the caller prior to the rifftInit call. The rifftCreate function also uses

rifftInit to initialize the dfr16_tRIFFTStruct that it dynamically allocates.

- DFR1_dfr16Cbitrev -The method bit-reverses the position of n complex data elements in the input vector, pointed to by pX.

The cbitrev function is used to convert normal, (linear), order complex vectors to bit-reversed complex vectors,

and vice-versa, for use in Fast Fourier Transform functions.

Module "Term1"     (bean Term)

- Term1_SendStr -Send string to terminal.

- Term1_SendNum -Send number to terminal.

- Term1_SendChar -Send char to terminal.

- Term1_MoveTo -Move cursor to position x, y. Range of coordinates can be from 1 to the size, which depends on the used

terminal. Position [1,1] represents the upper left corner.

Module "led1"      (bean BitIO)

- led1_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led1_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led2"      (bean BitIO)

- led2_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led2_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led3"      (bean BitIO)

- led3_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led3_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led4"      (bean BitIO)

- led4_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led4_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led5"      (bean BitIO)

- led5_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led5_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led6"      (bean BitIO)

- led6_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led6_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led7"      (bean BitIO)

- led7_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led7_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led8"      (bean BitIO)

- led8_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led8_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led9"      (bean BitIO)

- led9_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available only

if the direction = _[output]_ or _[input/output]_.

- led9_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led10"     (bean BitIO)

- led10_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- led10_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led11"     (bean BitIO)

- led11_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- led11_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led12"     (bean BitIO)

- led12_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- led12_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led13"     (bean BitIO)

- led13_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- led13_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led14"     (bean BitIO)

- led14_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- led14_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "led15"     (bean BitIO)

- led15_ClrVal -Clears (set to zero) the output value. It is equivalent to the [PutVal(FALSE)]. This method is available

only if the direction = _[output]_ or _[input/output]_.

- led15_SetVal -Sets (to one) the output value. It is equivalent to the [PutVal(TRUE)]. This method is available only if the

direction = _[output]_ or _[input/output]_.

Module "Cpu"       (bean Cpu)

- Cpu_EnableInt -Enables all maskable interrupts

- Cpu_DisableInt -Disables all maskable interrupts

- Cpu_SetWaitMode -Sets low power mode - Wait mode.  For more information about the wait mode see this CPU documentation.

Release from wait mode: Reset or interrupt

- Cpu_SetStopMode -Sets low power mode - Stop mode.  For more information about the stop mode see this CPU documentation.

- Cpu_Delay100US -This method realizes software delay. The length of delay is at least 100 microsecond multiply input

parameter [us100]. As the delay implementation is not based on real clock, the delay time may be increased by

interrupt service routines processed during the delay. Adding read wait states for external program memory can

cause delay extension as well. The method is independent on selected speed mode.

- Cpu_EnablePLL -Initializes and enables the PLL as configured in the bean inspector for the [high speed mode only]. This

method is available only if the property 'Enable PLL after reset' is set to 'no'. User is responsible for

enabling the PLL using this method. See also hint of the property <Enable PLL after reset> for details and

possible problems.

===================================================================================