# Atari 5200 32K PCB Rev. A

16K (27128) Conversion

To use a 16KB (27128) EPROM as opposed to a 32KB (27256) for which this PCB was designed, a simple cut and 2 wire additions are necessary.

- Do not populate the 7408 chip

- Cut trace between 7408 pin 1 and 27256 pin 27

- Add wire between 7408 pin 1 and 7408 pin 3

- Add wire between 27256 pin 27 to 27256 pin 28